4x1 超高清无缝切换器，KVM控制器

V1.0

警告

- 请勿将本设备暴露在雨水、湿气和滴水中

- 请使用随机配送的附件

- 在闪电雷暴天气下，请拔掉电源

内容

1.	特点	3

2.	面板	4

3.	EDID 处理	6

4.	视频和音频	6

5.	键盘热键	7

6.	规格	7

7.	包装	7

8.	RS232 命令	7

System command	8

Switching command	9

Output command	9

Audio command	10

EDID command	11

Status command	11

介绍

本产品是一款4x1超高清无缝切换器,支持4台电脑的DP视频信号及KVM控制，支持麦克风及左右声道模拟音频的输入输出，支持USB 2.0及键盘鼠标，支持4路RS232串口信号的切换选择

- 特点

- 4路电脑的KVM信号输入，支持DP1.2

- 不同的视频源间无缝切换， 最大视频分辨率 3840x2160p60

- 每个KVM设备支持两路USB HOST输入， 其中HOST 1用来接键盘鼠标，HOST 2用来接USB 2.0设备

- 1路麦克风输入，4路麦克风输出

- 4路DP视频信号/RS232信号/USB信号/左右声道音频信号同步切换

- KVM切换时，键盘鼠标无需重新建立连接，无卡顿或等待时间

- 可以通过RS232串口，网口，前面板以及键盘热键进行KVM切换

- 支持双电源冗余功能

- 面板

前面板

| 名称 | 说明 |
| --- | --- |
| MONITOR 灯 | 显示设备正常接入本设备时灯亮 |
| POWER 2灯 | 电源2接入时灯亮 |
| POWER 1灯 | 电源1接入时灯亮 |
| COM 灯 | 网口或串口与本设备进行通信时灯闪烁 |
| IN 1 | 输入1选择带灯按键及两个LED指示灯，当选择这路信号时，按键内部的灯会点亮；当这一路输入有信号时，SIGNAL灯会点亮; 当这一路的HOST 1有接入时（用来接键盘或鼠标），HOST 1灯会点亮 长按IN 1按键3秒，切换输出分辨率至2048x2048p60 |
| IN 2 | 输入2选择带灯按键及两个LED指示灯，当选择这路信号时，按键内部的灯会点亮；当这一路输入有信号时，SIGNAL灯会点亮; 当这一路的HOST 1有接入时（用来接键盘或鼠标），HOST 1灯会点亮 长按IN 2按键3秒，切换输出分辨率至3840x2160p60 |
| IN 3 | 输入3选择带灯按键及两个LED指示灯，当选择这路信号时，按键内部的灯会点亮；当这一路输入有信号时，SIGNAL灯会点亮; 当这一路的HOST 1有接入时（用来接键盘或鼠标），HOST 1灯会点亮 长按IN 3按键3秒，切换输出分辨率至1920x1080p60 |
| IN 4 | 输入4选择带灯按键及两个LED指示灯，当选择这路信号时，按键内部的灯会点亮；当这一路输入有信号时，SIGNAL灯会点亮; 当这一路的HOST 1有接入时（用来接键盘或鼠标），HOST 1灯会点亮 长按IN 4按键3秒，恢复出厂设置 |
| USB | 用来接入USB设备，支持到USB2.0 |

后面板

| 名称 | 说明 |
| --- | --- |
| LAN,RS232-CTL | 网口控制和RS232串口控制， 波特率9600 默认网络参数如下，可通过指令修改： IP Address: 192.168.0.247 SUBMASK: 255.255.255.0 GATEWAY:192.168.0.1 NETPORT: 2000 |
| MIC IN | 麦克风输入，仅支持MIC-LINE信号， 不支持驻极体等麦克风 |
| MIC OUT | 麦克风输出，信号强度为MIC-LINE 输出 |
| RS232 OUT | DB9公头,4路RS232信号经过4选1切换后接入这个口 |
| Keyboard/Mouse | 接入键盘鼠标 |
| HOST 1,2 | 接入PC的两个USB口，HOST 1用来做键盘鼠标控制用，HOST 2用来接入其它USB设备 |
| L/R IN | 左右声道音频输入 |
| MIC OUT | 麦克风输出 |
| RS232 IN | DB9母座， 4个连接PC的RS232口 |
| UPDATE | USB升级口，可接入U盘升级 |
| POWER 1,2 | 双电源冗余输入， 可接入其中一路或两路同时接入 |

- EDID 处理

用户可以通过串口指令选择以下EDID模式

| Number | EDID mode | Number | EDID mode |
| --- | --- | --- | --- |
| 1 | 4K60-2.0CH | 8 | 1600x1200 |
| 2 | 2048x2048 | 9 | 1440x900 |
| 3 | 4K30-2.0CH | 10 | 1360x768 |
| 4 | 1080P-2.0CH | 11 | 1280x1024 |
| 5 | 720P | 12 | 1024x768 |
| 6 | 1920x1200 | 13 | AUTO |
| 7 | 1680x1050 | 14 | USER |

- 视频和音频

本设备支持最大到3840x2160p60视频分辨率输入，支持以下可选的分辨率输出

| Number | Output Resolution | Number | Output Resolution |
| --- | --- | --- | --- |
| 1 | 4096x2160p 60Hz | 8 | 1920x1080p 60Hz |
| 2 | 4096x2160p 50Hz | 9 | 1920x1080p 50Hz |
| 3 | 3840x2160p 60Hz | 10 | 1360x768p 60Hz |
| 4 | 3840x2160p 50Hz | 11 | 1280x800p 60Hz |
| 5 | 3840x2160p 30Hz | 12 | 1280x720p 60Hz |
| 6 | 2048x2048p 60Hz | 13 | 1280x720p 50Hz |
| 7 | 1920x1200p60Hz RB | 14 | 1024x768 60Hz |

支持DP信号内嵌音频解嵌输出，以及外部模拟音频加嵌输入， DP输出和模拟音频输出可选外部音频和DP内嵌音频，也可以分别取自身的音频

- 键盘热键

Ctrl + Ctrl + 1表示切换KVM信号至IN 1，同理可切换至IN 2,3,4

Ctrl + Ctrl + 0表示返回上一次信号源

这里的 “+”表示操作上的先后顺序

- 规格

| 带宽 | 594MHz (18Gbps), DP1.2 |
| --- | --- |
| 输入口 | 4路DP,1路麦克风，4路RS232, 2x4路USB-HOST,4路模拟音频 |
| 输出口 | 1路DP， 4路麦克风，1路RS232，1路USB 2.0, 1路键盘和鼠标 |
| 交流电源 | 110-220VAC |
| 工作温度 | 0 to +40°C (+32 to +104 °F) |
| 工作湿度 | 10 to 70 % RH (non-condensing) |
| ESD | Air: ± 8KV, Contact: ± 4KV, |
| 尺寸 | L430 x W220 x H67 mm |
| 重量 | 7kg |

- 包装

| 内容 | 数量 |
| --- | --- |
| 切换器 | 1 |
| 电源线 | 2 |

- RS232 命令

注意: 所有的命令都以 SET 或 GET开始, 以回车符CR结束， 代表回车符CR(ASCII编码值为0x0D)

Baud Rate: 9600, Data Bits:8, Parity: None Stop Bits:1

System command

| Command | Details |
| --- | --- |
| GET HELP | Get the Commands list |
| SET RESET | Recover to default setting |
| GET VERSION | Get firmware version Return: VERSION w (w is version number) |
| GET MODEL TYPE | Get device model name Return: MODEL TYPE KVM41DP |
| SET BAUDRATE w | w is 9600, 19200, 38400,57600 or 115200 Return: BAUDRATE w |
| GET BAUDRATE w | Return: BAUDRATE w |
| GET POWER1 | Return: POWER1 ON or OFF |
| GET POWER2 | Return: POWER2 ON or OFF |
| SET IP ADDRESS w | For example: SET IP ADDRESS 192.168.0.247 Return: IP ADDRESS w |
| GET IP ADDRESS | Return: IP ADDRESS w |
| SET SUBMASK w | For example: SET SUBMASK 255.255.255.0 Return: SUBMASK w |
| GET SUBMASK | Return: SUBMASK w |
| SET GATEWAY w | For example: SET GATEWAY 192.168.0.1 Return: GATEWAY w |
| GET GATEWAY | Return: GATEWAY w |
| SET NETPORT w | For example: SET NETPORT 2000 Return: NETPORT w |
| GET NETPORT | Return: NETPORT w |
| SET NETWORK-INFO IP PORT SUBMASK GATEWAY | For Example: SET NETWORK-INFO 192.168.0.247 2000 255.255.255.0 192.168.0.1 Return: NETWORK-INFO 192.168.0.247 2000 255.255.255.0 192.168.0.1 |
| GET NETWORK-INFO | Return: NETWORK-INFO IP PORT SUBMASK GATEWAY |

Switching command

| Commands | Details |
| --- | --- |
| SET IN SOURCE w | w is one of the following: IN1, IN2, IN3, IN4, Return: IN SOURCE w |
| GET IN SOURCE | Get current input channel selection information Return: IN SOURCE w |
| GET IN RESOLUTION | Get current input resolution Return: IN RESOLUTION w (w is input resolution) |

Output command

| Commands | Details |
| --- | --- |
| SET OUT RESOLUTION w | w is one of the following, default: Auto 4096x2160p60, 4096x2160p50, 3840x2160p60, 3840x2160p50, 3840x2160p30, 2048x2048p60, 1920x1200p60RB, 1920x1080p60, 1920x1080p50, 1360x768p60, 1280x800p60, 1280x720p60, 1280x720p50, 1024x768p60, Auto, Return: OUT RESOLUTION w |
| GET OUT RESOLUTION | Get current output resolution setting Return: OUT RESOLUTION w |
| SET OUT VKA w | Set video keep alive mode w is BLUESCREEN or BLACKSCREEN. Default BLACKSCREEN. It is for no signal display Return: OUT VKA w |
| GET OUT VKA | Return: OUT VKA w |
| SET OUT ITC w | w is ON or OFF, default OFF Return: OUT ITC w Suggest OFF for video display and ON for PC especially desktop display, default OFF |
| GET OUT ITC | Return: OUT ITC w |

Audio command

| Commands | Details |
| --- | --- |
| SET AUDIO SRC w | Audio selection, w is EACH，LR or INNER, default EACH EACH means DP output with input DP INNER audio, LR output with LR INPUT audio |
| GET AUDIO SRC | Return: AUDIO SOURCE w |

EDID command

The following commands are used to set EDID mode for the inputs

| Commands | Details |
| --- | --- |
| SET IN EDIDMODE w | w is one of the following: 4K60-2.0, 2048x2048, 4K30-2.0, 1080p60-2.0, 1920x1200, 1680x1050, 1600x1200, 1440x900, 1360x768, 1280x1024, 1024x768, 720p, AUTO,USER Default: AUTO Return: IN EDIDMODE w |
| SET EDID-USER w | Switcher can only support 256 bytes EDID-USER data. w is 256 bytes EDID data. Return: EDID-USER OK |
| GET IN EDIDMODE | Return: IN EDIDMODE w |

Status command

The following commands are used to get the status of the switcher

| Commands | Details |
| --- | --- |
| GET IN STATUS | Get the switcher input status(VALID/INVALID) Return: IN STATUS IN1 VALID IN STATUS IN2 INVALID IN STATUS IN3 INVALID IN STATUS IN4 VALID |
| GET OUT STATUS | Get the switcher output status(VALID/INVALID) Return: OUT STATUS OUT VALID |
| GET POWER STATUS | Get the switcher power status(VALID/INVALID) Return: POWER STATUS POWER1 VALID POWER STATUS POWER2 INVALID |
| GET HOST STATUS | Get the switcher host status(VALID/INVALID) Return: HOST STATUS IN1-HOST1 VAILD HOST STATUS IN2-HOST1 INVALID HOST STATUS IN3-HOST1 INVALID HOST STATUS IN4-HOST1 INVALID |