UHD 无缝切换器/画面分割器

版本：V1.5

警告

- 请勿将本设备暴露在雨水、湿气和滴水中

- 请使用随机配送的附件

- 在闪电雷暴天气下，请拔掉电源

- 本说明书仅供参考， 如有更改恕不预先通知

内容

1. 特点	3

2. 面板布局	4

3. 遥控器	5

4. EDID 和HDCP 处理	5

5. 视频和音频	6

6. 多窗口	6

7. OSD 菜单导航	7

8. 规格	8

9. 包装	9

10. RS232 命令	9

System command	9

Switching command, only available on SINGLE mode	9

Output command	10

Multiview command	11

Audio command	15

EDID command	16

介绍

4路HDMI 2.0输入/1路HDMI 2.0输出无缝切换/画面分割器可以将多大4路信号源同时在1个大屏上显示.

用户可以方便的通过前面板按键/遥控器和串口指令来进行操作

应用领域:

家庭影院; 视频会议; 安防监控; 演示广告; 教学系统; 金融分析; 电竞赛场; 医疗显示

- 特点

- 4x HDMI 输入, 1x HDMI 输出

- 支持 HDMI 2.0,HDCP 2.2, 支持最大分辨率到3840x2160@60

- 支持5种多窗口模式, SINGLE, PIP, PBP, 3xWIN, 4xWIN

- 单窗口无缝切换，多窗口快速切换

- 支持音量控制和音频独立选择

支持 LPCM, AC3, DD+, DTS, DTS-HD, 最大到 7.1 声道

- 支持OSD 菜单导航

- 支持多种 EDID 和HDCP 操作

- 面板布局

前面板

| Name | Description |
| --- | --- |
| AUTO, RESO | 按AUTO 键可以激活或关闭自动信源切换功能 该功能仅在单窗口模式下可以激活 长按该按键3秒，可循环切换输出分辨率（分辨率切换时，前面板的灯会有熄灭到点亮的指示） |
| HDMI 1, HDMI 2 HDMI 3, HDMI 4 | 短按进行信号源选择 长按这4个按键3秒，可分别进入Single, PIP, PBP或3xWIN |
| WINDOW | 按下该按键, 屏幕上将显示一个窗口的边框， 循环按该按键将选择不同的窗口画面，然后按HDMI 1,2,3或4键选择该窗口显示的信号源. 长按该按键3秒，可进入4xWIN显示模式 |

后面板

| Name | Description |
| --- | --- |
| HDMI out | HDMI 输出，最大到 3840x2160@60 |
| AUDIO OUTPUT | 3.5mm L+R 输出, 20Hz ~ 20kHz, 1.5Vrms max; Toslink-optical 输出 |
| INPUTs | HDMI 1, HDMI 2, HDMI 3, HDMI 4 |
| RS232 control | 默认：Baud rate 9600, 8 data bits, 1 stop bit, no parity Baud rate 可以通过OSD菜单更改 PIN 2 TX ( Matrix → PC) PIN 3 RX (Matrix ←PC ) PIN 5 Ground |
| 12V | 12V 电源适配器插入 |

侧面板

侧面板上USB-A 口  用作U盘升级用， USB-B 口  用作PC控制用

- 遥控器

| Number | Description |
| --- | --- |
| 1 | 返回 |
| 2 | 输入 |
| 3 | OSD 菜单导航 |
| 4 | 窗口模式选择 |
| 5 | 音源选择 |
| 6 | 预留 |

- EDID 和HDCP 处理

用户可以通过串口命令或OSD菜单选择以下EDID模式

| Number | EDID mode | Number | EDID mode |
| --- | --- | --- | --- |
| 1 | 4K60-2.0CH | 10 | 1600x1200 |
| 2 | 4K60-5.1CH | 11 | 1440x900 |
| 3 | 4K30-2.0CH | 12 | 1360x768 |
| 4 | 4K30-5.1CH | 13 | 1280x1024 |
| 5 | 1080P-2.0CH | 14 | 1024x768 |
| 6 | 1080P-5.1CH | 15 | AUTO |
| 7 | 720P | 16 | 4K60-7.1CH |
| 8 | 1920x1200 | 17 | 4K30-7.1CH |
| 9 | 1680x1050 | 18 | 1080P-7.1CH |
|  |  | 19 | USER |

HDMI输出支持3种HDCP选项: FORCE-1.4, FORCE-2.2, FORCE-OFF

用户可以通过RS232命令选择

- 视频和音频

切换器支持最大输入分辨率3840x2160@60, 支持多种音频格式，比如 LPCM, AC3, DD+, DTS, DTS-HD, 最大到7.1 声道， 支持音频独立选择

用户可以控制LPCM格式音频的音量

支持以下输出分辨率选择

| Number | Output Resolution | Number | Output Resolution |
| --- | --- | --- | --- |
| 1 | 4096x2160p 60Hz | 8 | 1920x1080p 60Hz |
| 2 | 4096x2160p 50Hz | 9 | 1920x1080p 50Hz |
| 3 | 3840x2160p 60Hz | 10 | 1360x768p 60Hz |
| 4 | 3840x2160p 50Hz | 11 | 1280x800p 60Hz |
| 5 | 3840x2160p 30Hz | 12 | 1280x720p 60Hz |
| 6 | 3840x2160p 25Hz | 13 | 1280x720p 50Hz |
| 7 | 1920x1200p60Hz RB | 14 | 1024x768 60Hz |

- 多窗口

支持5种多窗口模式

SINGLE, PIP, PBP, 3xWIN, 4xWIN

对于不同的多窗口模式，用户可以进行不同的操作

SINGLE: 信源选择

PIP: 信源选择, 子画面大小和位置

PBP, 3xWIN, 4xWIN: 信源选择, 显示比例,模式选择

默认布局如下

用户可以通过串口命令或OSD菜单进行选择

- OSD 菜单导航

总共6个按键进行OSD菜单导航, 菜单/返回/上/下/左/右

菜单类容如下：

| 输出设置 | 分辨率 | 3840x2160p60 | 3840x2160p60,… |
| --- | --- | --- | --- |
|  | 无信号设置 | 黑屏 | 黑屏,蓝屏 |
|  | 4K自动 | 开 | 开,关 |
|  | ITC | 关 | 开,关 |
| Multiview | 单画面 | 输入选择 | HDMI1, … |
|  | PIP | 窗口1选择 | HDMI1,… |
|  |  | 窗口2选择 | HDMI1,… |
|  |  | PIP 位置 | 右下,右上,左下,左上 |
|  |  | PIP 大小 | 大,中,小 |
|  | PBP | 窗口1选择 | HDMI1,… |
|  |  | 窗口2选择 | HDMI1,… |
|  |  | 模式 | 1, 2 |
|  |  | 画面比例 | 全屏, 16:9 |
|  | 3xWIN | 窗口1选择 | HDMI1,… |
|  |  | 窗口2选择 | HDMI1,… |
|  |  | 窗口3选择 | HDMI1,… |
|  |  | 模式 | 1, 2 |
|  |  | 画面比例 | 全屏, 16:9 |
|  | 4xWIN | 窗口1选择 | HDMI1,… |
|  |  | 窗口2选择 | HDMI1,… |
|  |  | 窗口3选择 | HDMI1,… |
|  |  | 窗口4选择 | HDMI1,… |
|  |  | 模式 | 1, 2 |
|  |  | 画面比例 | 全屏,16:9 |
| 声音设置 | 音源选择 | 窗口1 | 窗口1,HDMI1,… |
|  | 音量 | 100 | 0..100 |
|  | 静音 | 关 | 开, 关 |
| 系统设置 | Language/语言 | English | English, 中文 |
|  | EDID | 4K60-2.0 | 4K60-2.0,… |
|  | 波特率 | 9600 | 9600,19200,38400, 57600，115200 |
|  | 复位 |  |  |
|  | 固件版本 |  | Read only |

请注意, 关于ITC 的设置, 建议视频信源时设置为关，PC界面尤其是电脑桌面时用开，默认关

- 规格

| 带宽 | 594MHz (18Gbps), HDMI 2.0, HDCP2,2 |
| --- | --- |
| 音频格式 | LPCM, AC3, DD+, DTS, DTS-HD Up to 7.1 channel |
| 输入口 | 4x HDMI |
| 输出口 | 1x HDMI 1x 3.5mm LR audio 1x Toslink digital audio |
| 电源 | 12V/3A ,12W max |
| 工作温度 | 0 to +40°C (+32 to +104 °F) |
| 工作湿度 | 10 to 70 % RH (non-condensing) |
| ESD | Air: ± 8KV, Contact: ± 4KV, |
| 尺寸 | L225 x W100 x H25 mm |
| 重量 (Main Unit) | 0.75kg |

- 包装

| 项目 | Quantity |
| --- | --- |
| 主机 | 1 |
| 12V/3A 适配器 | 1 |
| 用户手册 | 1 |
| 遥控器 | 1 |

- RS232 命令

Note: All the commands begin with SET or GET, end with Carriage Return (CR).

Represents Carriage Return (CR).

All return messages are always end with CR.

System command

| Command | Details |
| --- | --- |
| GET HELP | Get the Commands list |
| SET RESET | Recover to default setting |
| GET VERSION | Get firmware version Return: VERSION w (w is version number) |
| SET BAUDRATE w | w is 9600, 19200, 38400,57600 or 115200 Return: BAUDRATE w |
| GET BAUDRATE w | Return: BAUDRATE w |

Switching command, only available on SINGLE mode

| Commands | Details |
| --- | --- |
| SET AUTO SWITCH w | w is ON or OFF, default OFF Return: AUTO SWITCH w |
| GET AUTO SWITCH | Return: AUTO SWITCH w |
| SET IN SOURCE w | w is one of the following: HDMI1, HDMI2, HDMI3, HDMI4 Return: IN SOURCE w |
| GET IN SOURCE | Get current input channel selection information Return: IN SOURCE w |
| GET IN RESOLUTION | Get current input resolution Return: IN RESOLUTION w (w is input resolution) |
| GET IN STATUS | Get status of all input ports x is HDMI1…HDMI4 Return: IN STATUS x VALID(or INVALID) If input port is vaild, Return: IN STATUS x InputRes ColorSpace ColorDepth |

Output command

| Commands | Details |
| --- | --- |
| SET OUT RESOLUTION w | w is one of the following, default: 1920x1080p60 4096x2160p60, 4096x2160p50, 3840x2160p60, 3840x2160p50, 3840x2160p30, 3840x2160p25, 1920x1200p60RB, 1920x1080p60, 1920x1080p50, 1360x768p60, 1280x800p60, 1280x720p60, 1280x720p50, 1024x768p60， AUTO, USER Return: OUT RESOLUTION w |
| GET OUT RESOLUTION | Get current output resolution setting Return: OUT RESOLUTION w |
| SET RESO-USER Width Height | Set user define output resolution, Width is horizontal active pixels Height is vertical active lines For user define output resolution,the frame rate is always 60Hz Return: RESO-USER Width Height |
| GET RESO-USER | Return: RESO-USER Width Height |
| SET OUT HDCP w | w is one of following, default FORCE-OFF FORCE-1.4,FORCE-2.2,FORCE-OFF Return: OUT HDCP w |
| GET OUT HDCP | Return: OUT HDCP w |
| SET OUT VKA w | w is BLUESCREEN or BLACKSCREEN. Default BLACKSCREEN. It is for no signal display Return: OUT VKA w |
| GET OUT VKA | Return: OUT VKA w |
| SET OUT ITC w | w is ON or OFF, default OFF Return: OUT ITC w |
| GET OUT ITC | Return: OUT ITC w |

Multiview command

| Commands | Details |
| --- | --- |
| SET MULTIVIEW w | Select one Multiview mode for current display w is one of the following, default SINGLE SINGLE , PIP , PBP , 3xWIN , 4xWIN Return: MULTIVIEW w |
| GET MULTIVIEW | Get the current Multiview mode Return: MULTIVIEW w |
| SET WINDOWx IN y | Select one input for one display window for the current Multiview mode. x is one of 1, 2, 3 or 4 y is one of HDMI1, HDMI2, HDMI3, HDMI4 Return: WINDOWx IN y |
| GET WINDOWx IN | This command to get which is the input source for one display window for the current Multiview mode Return: WINDOWx IN y |

| SET FREEZE-WINx w | Freeze the display window,x is one of 1, 2, 3 ,4 or ALL, w is ON or OFF Return: FREEZE-WINx w |
| --- | --- |
| GET FREEZE-WINx | x is one of 1, 2, 3 ,4. Return: FREEZE-WINx w (w is ON or OFF) |

| SET PIP POS w | This command to select the PIP sub window position. w is one of the following, default RightBottom LeftTop, LeftBottom, RightTop, RightBottom, USER Return: PIP POS w |
| --- | --- |
| GET PIP POS | This command to get the PIP sub window position Return: PIP POS w |
| SET PIP SIZE w | This command to select the PIP sub window size. w is one of the following, default LARGE SMALL,MIDDLE, LARGE, USER Return: PIP SIZE w |
| GET PIP SIZE | Return: PIP SIZE w |
| SET PIP USER HStart VStart HSize VSize | Return: PIP USER HStart VStart HSize VSize This command allows users to customize a PIP layout include sub window position and size. This customized PIP layout will replace other pre-defined PIP modes (such as LeftTop,LARGE) and display on the screen After the user enters SET PIP POS or SET PIP SIZE command,the PIP USER will become invalid Please note HStart plus HSize less than or equal to 101 VStart plus VSize less than or equal to 101 |
| GET PIP USER | Return: PIP USER HStart VStart HSize VSize |
| SET PBP MODE w | Set the PBP display mode w is one of 1,2 or 3, default 1 Return: PBP MODE w Please note for PBP mode 3, window2 can capture part of the input image area. It is main used for presenter show when work with conference camera situations The capture area can be defined by SET PBP-PRESENTER command |
| GET PBP MODE | Return: PBP MODE w |
| SET PBP ASPECT w | Set the PBP window display aspect w is FULL or 16:9, default FULL Return: PBP ASPECT w |
| GET PBP ASPECT | Return: PBP ASPECT w |
| SET PBP-PRESENTER HStart VStart HSize VSize | Set window 1 capture area for PBP mode 3 This command only valid when the switcher already work on PBP mode 3 Return: PBP-PRESENTER HStart VStart HSize VSize Default HStart 38, VStart 13, HSize 25, VSize 75 Please note HStart plus HSize less than or equal to 101 VStart plus VSize less than or equal to 101 |
| GET PBP-PRESENTER | Return: PBP-PRESENTER HStart VStart HSize VSize |
| SET 3xWIN MODE w | Set the 3xWIN display mode w is one of 1,2,3 or 4, default 1 Return: 3xWIN MODE w |
| GET 3xWIN MODE | Return: 3xWIN MODE w |
| SET 3xWIN ASPECT w | Set the 3xWIN window display aspect w is FULL or 16:9, default FULL Return: 3xWIN ASPECT w |
| GET 3xWIN ASPECT | Return: 3xWIN ASPECT w |
| SET 4xWIN MODE w | Set the 4xWIN display mode w is 1 or 2 ,default 1 Return: 4xWIN MODE w |
| GET 4xWIN MODE | Return: 4xWIN MODE w |
| SET 4xWIN ASPECT w | Set the 4xWIN window display aspect w is FULL or 16:9, default FULL Return: 4xWIN ASPECT w |
| GET 4xWIN ASPECT | Return: 4xWIN ASPECT w |
| GET MULTIVIEW-SYNC | Return Multiview layout information |
| SET SAVE SCENE w | Save current display scene w is 1, 2,…20 Return: SAVE SCENE w |
| SET LOAD SCENE w | Load display scene w is 1, 2,…20 Return: LOAD SCENE w |

Audio command

| Commands | Details |
| --- | --- |
| SET AUDIO SOURCE w | w is one of the following: WIN1, HDMI1, HDMI2, HDMI3, HDMI4 Return: AUDIO SOURCE w |
| GET AUDIO SOURCE | Return: AUDIO SOURCE w |
| SET AUDIO VOL+ | Increase audio out volume Return: AUDIO VOL w (w is the volume value) |
| SET AUDIO VOL- | Decrease audio out volume Return: AUDIO VOL w (w is the volume value) |
| SET AUDIO VOL w | Set audio volume value w is 0,1…,or 100, default 100 For example: SET AUDIO VOL 100 Return: AUDIO VOL w |
| GET AUDIO VOL | Return: AUDIO VOL w |
| SET AUDIO-MUTE w | Mute or unmute audio output Here w is ON or OFF, default OFF Return: AUDIO-MUTE w |
| GET AUDIO-MUTE | Return: AUDIO-MUTE w |

EDID command

The following commands are used to set EDID mode for the inputs

| Commands | Details |
| --- | --- |
| SET IN EDIDMODE w | w is one of the following: 4K60-2.0, 4K60-5.1, 4K60-7.1, 4K30-2.0, 4K30-5.1, 4K30-7.1, 1080p60-2.0,1080p60-5.1, 1080p60-7.1,1920x1200, 1680x1050, 1600x1200, 1440x900, 1360x768, 1280x1024, 1024x768, 720p, AUTO,USER Default: 4K60-2.0 Return: IN EDIDMODE w |
| SET EDID-USER w | Switcher can only support 256 bytes EDID-USER data. w is 256 bytes EDID data. Return: EDID-USER OK |
| GET IN EDIDMODE | Return: IN EDIDMODE w |